SWITCH CONFIG

Switch#show running-config

Switch#config t

Switch(config)#hostname SWITCH1

SWITCH1(config)#exit

SWITCH1#Exit

SWITCH1#reload

Switch#config terminal

SWITCH1#copy running-config  startup-config

CONSOLE PASSWORD

SWITCH1(config)#line console 0

SWITCH1(config-line)#password cisco1

SWITCH1(config-line)#login

SWITCH1(config-line)#exit

Privileged mode password

SWITCH1#config terminal

SWITCH1(config)#enable password cisco2

SWITCH1(config)#exit

To remove priviledge mode password

SWITCH(config)#no enable password

IP address to Vlan 1 in the Switch:

SWITCH(config)#no ip domain-lookup

SWITCH(config)#interface vlan 1

SWITCH(config-if)#ip address 192.168.10.1 255.255.255.0

SWITCH(config-if)#no shutdown

SWITCH(config-if)exit

Assign Remote(Telnet) password:

SWITCH#config terminal

SWITCH(config)#line vty ?

SWITCH(config)#line vty 0 15

SWITCH(config-line)#password cisco3

SWITCH(config-line)#exit

C:\>ipconfig /all

C:\>ping 192.168.10.1

C:\>telnet 192.168.10.1

Banner and Secret Password

SWITCH(config)#banner motd ?

LINE  c banner-text c, where 'c' is a delimiting character

you can replace c with # also

SWITCH(config)#banner motd c Networking Training c

SWITCH(config)#enable secret cisco4

SWITCH(config)#exit

ROUTER CONFIG

Router(config)#hostname Router1

Router1#reload

Router1#copy running-config  startup-config

Assigning Console password:

Router1(config)#line console 0

Router1(config-line)#password cisco1

Router1(config-line)#login

Router1(config-line)#exit

Privileged mode password:

Router1(config)#enable password cisco2

Router1(config)#exit

Router(config)#no enable password

Assigning IP address to GigabitEthernet0/0 in the Router:

Router1(config)#interface GigabitEthernet0/0

Router1(config-if)#ip address 192.168.10.10 255.255.255.0

Router(config-if)#no shutdown

Router1(config-if)exit

Assign Remote(Telnet) password:

Router1(config)#line vty ?

<0-15>  First Line number

Router1(config)#line vty 0 15

Router1(config-line)#password cisco3

Router1(config-line)#exit

C:\>ipconfig

C:\>ping 192.168.10.10

C:\>telnet 192.168.10.10

Banner and Secret Password

Router1(config)#banner motd ?

LINE  c banner-text c, where 'c' is a delimiting character

you can replace c with # also

Router1(config)#banner motd c Networking Training c

Router1(config)#enable secret cisco4

Router1(config)#exit

Accessing Router Remotely using SSH

Router1(config)#ip domain-name example.com

Router1(config)#username admin password cisco9

Router1(config)#crypto key generate rsa

The name for the keys will be: Router1.example.com

How many bits in the modulus [512]: 2048

Router1(config)#line vty 0 15

Router1(config-line)#login local

Router1(config-line)#transport input ssh

Router1(config-line)#exit

C:\>ssh -l admin 191.168.10.10

ROUTER BACKUP

Router1#copy running-config  startup-config

Router1#show startup-config

Router#copy startup-config tftp:

Address or name of remote host []? 192.168.10.5

Destination filename [Router-confg]?

Router#show flash:

Router#copy flash: tftp:

Source filename []? c2900-universalk9-mz.SPA.151-4.M4.bin

Address or name of remote host []? 192.168.10.5

Destination filename [c2900-universalk9-mz.SPA.151-4.M4.bin]?

Verify availability of both the files in the TFTP server

delete IOS file and restore from TFTP server

Router#show flash:

Router#delete flash:

Delete filename []?c2900-universalk9-mz.SPA.151-4.M4.bin

Delete flash:/c2900-universalk9-mz.SPA.151-4.M4.bin? [confirm]       ==>Press enter key

Router#copy tftp: flash:

Address or name of remote host []? 192.168.10.5

Source filename []? c2900-universalk9-mz.SPA.151-4.M4.bin

Router#show flash:

Router#copy tftp: startup-config

Router1#show startup-config

RESTORING IOS FROM TFTP TO ROUTER

Router#copy flash: tftp:

Example:

Source filename []? c2900-universalk9-mz.SPA.157-3.M.bin

Address or name of remote host []? 192.168.10.2

Once the IOS file is copied to the TFTP server, you can find the IOS image on the TFTP server as

shown in the picture below.

Now delete the IOS file from the router

Router#show flash:

Router#delete flash:

Router#show Flash:

Router#reload

It will enter ROMMON mode because of the missing IOS file.

rommon 13 &gt;tftpdnld

Enter the required values in the given format

Note: all commands are case sensitive

rommon 13 &gt; IP_ADDRESS=192.168.10.1

rommon 14 &gt; IP_SUBNET_MASK=255.255.255.0

rommon 15 &gt; DEFAULT_GATEWAY=0.0.0.0

rommon 16 &gt; TFTP_SERVER=192.168.10.2

rommon 17 &gt; TFTP_FILE=c2900-universalk9-mz.SPA.151-4.M4.bin

rommon 18 &gt; TFTPDNLD

types y

romom19&gt;reset

Invoke this command for disaster recovery only.

PASSWORD RECOVERY

router(config)#enable secret cisco1

router(config)#line console 0

router(config-line)#password cisco2

router(config-line)#login

router(config-line)#exit

router(config)#line vty 0 15

router(config-line)#password cisco3

router(config-line)#login

router(config-line)#exit

router(config)#exit

router#

router#copy running-config startup-config

router#show startup-config

router#reload

Now you have forgotten password

manually switch off and switch on the Router

presss CTRL+c while restarting

rommon 1 >

rommon 1 > confreg 0x2142

rommon 2 > reset

press yes

after restart press no

router>

router>enable

router#

router#show running-config

router#show startup-config

router# copy startup-config running-config

router#

router#config t

router(config)#enable secret cisco4

router(config)#line console 0

router(config-line)#password cisco5

router(config-line)#login

router(config-line)#exit

router(config)#line vty 0 15

router(config-line)#password cisco6

router(config-line)#login

router(config-line)#exit

router(config)#exit

router#

router#show running-config

router#copy running-config startup-config

router#

router#config t

router(config)#

router(config)# config-register 0x2102

router(config)#exit

router#reload

after restart

login with new password

ROUTING

Configure default Route

R1#show ip route

R1(config)#ip route 0.0.0.0 0.0.0.0 192.168.10.2

R1#show ip route

STATIC ROUTING

R1(config)#ip route 172.16.0.0 255.255.192.0 192.168.10.2

R2(config)#ip route 172.16.0.0 255.255.192.0 192.168.12.66

R2(config)#ip route 10.0.0.0 255.240.0.0 192.168.10.1

R3(config)ip route 10.0.0.0 255.240.0.0 192.168.12.65

RIP

R1(config)#Router RIP

R1(config-router)#version 2

R1(config-router)#network 10.0.0.0

R1(config-router)#network 192.168.10.0

R2(config)#Router RIP

R2(config-router)#version 2

R2(config-router)#network 192.168.10.0

R2(config-router)#network 192.168.12.64

R3(config)#Router RIP

R3(config-router)#version 2

R3(config-router)#network 192.168.12.64

R3(config-router)#network 172.16.0.0

Remove RIP configuration from the Routers

R1(config)#router rip

R1(config-router)#no version 2

R1(config-router)#no network 10.0.0.0

R1(config-router)#no network 192.168.10.0

R1(config-router)#exit

EIGRP

R1(config)#router eigrp 1

R1(config-router)#network 10.0.0.0 0.15.255.255

R1(config-router)#network 192.168.10.0 0.0.0.3

R1#show ip route

R2(config)#router eigrp 1

R2(config-router)#network 192.168.10.0 0.0.0.3

R2(config-router)#network 192.168.12.64 0.0.0.3

R2#show ip route

R3(config)#router eigrp 1

R3(config-router)#network 192.168.12.64 0.0.0.3

R3(config-router)#network 172.16.0.0 0.0.63.255

R3#show ip route

Remove EIGRP from the routers

R1(config)#no router eigrp 1

R2(config)#no router eigrp 1

R3(config)#no router eigrp 1

OSPF

R1(config)#router ospf 1

R1(config-router)#router-id 1.1.1.1

R1(config-router)#network 10.0.0.0 0.15.255.255 area 0

R1(config-router)#network 192.168.10.0 0.0.0.3 area 0

R1#show ip route

R2(config)#router ospf 10

R2(config-router)#router-id 2.2.2.2

R2(config-router)#network 192.168.10.0 0.0.0.3 area 0

R2(config-router)#network 192.168.12.64 0.0.0.3 area 0

R2#show ip route

R3(config)#router ospf 30

R3(config-router)#router-id 3.3.3.3

R3(config-router)#network 192.168.12.64 0.0.0.3 area 0

R3(config-router)#network 172.16.0.0 0.0.63.255 area 0

R3#show ip route

ROUTE REDISTRIBUTION

Router1#config t

Router1(config)#

Router1(config)#interface loopback 0

Router1(config-if)#ip address 1.1.1.1 255.255.255.255

Router1(config-if)#exit

Router1(config)#

Router1(config)#router ospf 1

Router1(config-router)#network 50.0.0.0 0.255.255.255 area 15

Router1(config-router)#network 1.1.1.1 0.0.0.0 area 15

Router1(config-router)#redistribute rip subnets

Router1(config-router)#redistribute eigrp 20 subnets

Router1(config-router)#exit

Router1(config)#

Router1(config)#

Router1(config)#router rip

Router1(config-router)#version 2

Router1(config-router)#network 30.0.0.0

Router1(config-router)#redistribute eigrp 20 metric 2

Router1(config-router)#redistribute ospf 1 metric 2

Router1(config)#router eigrp 20

Router1(config-router)#network 40.0.0.0 0.255.255.255

Router1(config-router)#redistribute rip metric 1 1 1 1 1

Router1(config-router)#redistribute ospf 1 metric 1 1 1 1 1

Router1(config-router)#exit

Router1(config)#exit

Router1#

Router1#show ip route

ACL [1-99]

10.0.0.2 should not access LAN2(172.16.0.0/18)

Step1: Create ACL

R3(config)#ip access-list standard 1

R3(config-std-nacl)#deny host 10.0.0.2

R3(config-std-nacl)#permit any

R3(config-std-nacl)#exit

Step2: Apply ACL

R3(config)#interface g0/0

R3(config-if)#ip access-group 1 out

R3(config-if)exit

R3#show access-lists

ping from PC0(10.0.0.2) to pc2(172.16.0.2)

Remove Configuration

R3(config)#interface g0/0

R3(config-if)#no ip access-group 1 out

R3(config)#no ip access-list standard 1

EXTENDED ACL [100-199]

10.0.0.3 should not access HTTP from 172.16.0.4

R1(config)#access-list 100 deny tcp 10.0.0.3 0.0.0.0 172.16.0.4 0.0.0.0 eq www

R1(config)#access-list 100 permit ip any any

Applying ACL:

R1(config)#int g0/0

R1(config-if)#ip access-group 100 in

R1#show access-lists

NAMED ACL

R3(config)#ip access-list standard test1

R3(config-std-nacl)#deny host 10.0.0.2

R3(config-std-nacl)#permit any

R3(config-std-nacl)#exit

R3(config)#interface g0/0

R3(config-if)#ip access-group test1 out

R3#show access-lists

ping from 10.0.0.2 to 172.16.0.2 and 172.16.0.3

TELNET

No system in the network address 172.16.0.0/18 should access router R1 remotely using telnet(23)

R1(config)#enable secret cisco2

R1(config)#line vty 0 15

R1(config-line)#password cisco3

R1(config-line)#login

R1(config-line)#exit

R1(config)#

R1(config)#exit

try to access R1 from 172.16.0.2

R3(config)#access-list 110 deny tcp 172.16.0.0 0.0.63.255 192.168.10.1 0.0.0.0 eq telnet

R3(config)#access-list 110 deny tcp 172.16.0.0 0.0.63.255 10.0.0.1 0.0.0.0 eq telnet

R3(config)#access-list 110 permit ip any any

Applying ACL:

R3(config)#int g0/0

R3(config-if)#ip access-group 110 in

Accessing FTP Server

C:\>ftp 172.16.0.4

10.0.0.0/12 (no system from this network should access FTP) should not access FTP server(172.16.0.4)

R1(config)#access-list 102 deny tcp 10.0.0.0 0.15.255.255 172.16.0.4 0.0.0.0 eq FTP

R1(config)#access-list 102 permit ip any any

R1(config)#int g0/0

R1(config-if)#ip access-group 102 in

STATIC NAT

R1(config)#ip nat inside source static 10.0.0.2 100.1.1.1

R1(config)#ip nat inside source static 10.0.0.3 100.1.1.2

R1(config)#interface g0/0

R1(config-if)#ip nat inside

R1(config-if)#exit

R1(config)#interface serial 0/3/0

R1(config-if)#ip nat outside

R1(config-if)#exit

R1#show ip nat translations

Test: Ping from 172.16.0.2 pc to 10.0.0.2  -->you should get reply from 100.1.1.1

Test: Ping from 172.16.0.3 pc to 10.0.0.3  -->you should get reply from 100.1.1.2

Remove Static NAT configuration:

R1(config)#no ip nat inside source static 10.0.0.1 100.1.1.1

R1(config)#no ip nat inside source static 10.0.0.2 100.1.1.2

DYNAMIC NAT

R1(config)#access-list 1 permit 10.0.0.0 0.15.255.255

R1(config)#ip nat pool dynamicpool1 200.1.1.1 200.1.1.2 netmask 255.255.255.252

R1(config)#ip nat inside source list 1 pool dynamicpool1

R1(config)#interface g0/0

R1(config-if)#ip nat inside

R1(config-if)#exit

R1(config)#interface serial 1/0

R1(config-if)#ip nat outside

R1(config-if)#exit

R1#show ip nat translations

Remove Dynamic NAT:

R1(config)#no access-list 1 permit 10.0.0.0 0.15.255.255

R1(config)#no ip nat inside source list 1 pool dynamicpool1

R1(config)#no ip nat pool dynamicpool1 200.1.1.1 200.1.1.2 netmask 255.255.255.252

NAT OVERLOAD PAT

R1(config)#access-list 1 permit 10.0.0.0 0.15.255.255

R1(config)#ip nat pool pat1 12.1.1.1 12.1.1.1 netmask 255.255.255.255

R1(config)#ip nat inside source list 1 pool pat1 overload

R1(config)#interface g0/0

R1(config-if)#ip nat inside

R1(config-if)#exit

R1(config)#interface serial 1/0

R1(config-if)#ip nat outside

R1(config-if)#exit

R1#show ip nat translations

STP

Switch#show spanning-tree

ONE WILL BE THE ROOT

VLAN [ ROS / SVI ] 	INTERVLAN

create VLANs

Switch1(config)#vlan 2

Switch1(config-vlan)#name Sales

Switch1(config-vlan)#exit

Switch1(config)#vlan 3

Switch1(config-vlan)#name Marketing

Switch1(config-vlan)#exit

Switch1#show vlan

Assigning VLANs to respective interfaces

1 select interface

2 make them access port

3 assign VLAN number

Switch1(config)#interface fastEthernet 0/1

Switch1(config-if)#switchport mode access

Switch1(config-if)#switchport access vlan 2

Switch1(config-if)#exit

Switch1(config)#interface fastEthernet 0/2

Switch1(config-if)#switchport mode access

Switch1(config-if)#switchport access vlan 2

Switch1(config-if)#exit

Switch1(config)#interface fastEthernet 0/3

Switch1(config-if)#switchport mode access

Switch1(config-if)#switchport access vlan 3

Switch1(config-if)#exit

Switch1(config)#interface fastEthernet 0/4

Switch1(config-if)#switchport mode access

Switch1(config-if)#switchport access vlan 3

Switch1(config-if)#exit

Switch1#show vlan

Configuring port for Trunk mode which is connected to another switch/Router

Switch1(config)#interface fastEthernet 0/5

Switch1(config-if)#switchport mode trunk

Switch1(config-if)#exit

Switch1#show vlan

Switch1#show interfaces trunk

SWITCH2

CREATE VLAN

ASSIGN VLAN TO RESPECTIVE INTERFACE

PORT TRUNK THAT CONNECTS TO OTHER SWITCH

Verify the connection

ping from 92.168.10.1 to 192.168.10.2 , 192.168.10.5 , 192.168.10.6 you will get reply

ping from 92.168.10.1 to 192.168.10.3 , 192.168.10.4 , 192.168.10.7 and 192.168.10.8  you will get request timed out.

InterVLAN Configuration Using Router

Configuring port for Trunk mode which is connected to the Router

Switch1(config)#interface fastEthernet 0/6

Switch1(config-if)#switchport mode trunk

Switch1(config-if)#exit

Switch1#show vlan

Switch1#show interfaces trunk

create a subinterfaces on the router

Router#configure terminal

Router0(config)#interface fastEthernet 0/0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#interface fastEthernet 0/0.1

Router0(config-subif)#encapsulation dot1Q 2

Router0(config-subif)#ip address 192.168.10.10 255.255.255.0

Router0(config-subif)#exit

Router0(config)#interface fastEthernet 0/0.2

Router0(config-subif)#encapsulation dot1Q 3

Router0(config-subif)#ip address 192.168.11.10 255.255.255.0

Router0(config-subif)#exit

Router0#show running-config

Test the connection between different vlans

ping from 192.168.10.1 to 192.168.11.1 or any system part of 192.168.11.0/24 network, you will get reply

InterVLAN Configuration Using Layer3 Switch

create a VLANs

layer3Switch(config)#vlan 2

layer3Switch(config-vlan)#name Sales

layer3Switch(config-vlan)#exit

layer3Switch(config)#vlan 3

layer3Switch(config-vlan)#name Marketing

layer3Switch(config-vlan)#exit

layer3Switch#show vlan

assign VLAN to interfaces

layer3Switch(config)#interface fastEthernet 0/1

layer3Switch(config-if)#switchport mode access

layer3Switch(config-if)#switchport access vlan 2

layer3Switch(config-if)#exit

layer3Switch(config)#interface fastEthernet 0/2

layer3Switch(config-if)#switchport mode access

layer3Switch(config-if)#switchport access vlan 2

layer3Switch(config-if)#exit

layer3Switch(config)#interface fastEthernet 0/3

layer3Switch(config-if)#switchport mode access

layer3Switch(config-if)#switchport access vlan 3

layer3Switch(config-if)#exit

layer3Switch(config)#interface fastEthernet 0/4

layer3Switch(config-if)#switchport mode access

layer3Switch(config-if)#switchport access vlan 3

layer3Switch(config-if)#exit

layer3Switch#show vlan

assign ip address to SVI(Switch Virtual Interface)

layer3Switch(config)#interface vlan 2

layer3Switch(config-if)#ip address 192.168.10.10 255.255.255.0

layer3Switch(config-if)#no shutdown

layer3Switch(config-if)#exit

layer3Switch(config)#interface vlan 3

layer3Switch(config-if)#ip address 192.168.11.10 255.255.255.0

layer3Switch(config-if)#no shutdown

layer3Switch(config-if)#exit

enable ip routing in the switch

layer3Switch(config)#ip routing

layer3Switch(config)#exit

layer3Switch#show vlan

layer3Switch#show ip route

verify the connection

ping from 192.168.10.1 to 192.168.11.1, you will get reply

DTP

dynamic desirable 	dynamic desirable 	Trunk

dynamic desirable 	dynamic auto 		Trunk

dynamic auto 		dynamic auto 	              Access

trunk 			dynamic desirable 	Trunk

trunk 			dynamic auto 		Trunk

Access 			dynamic desirable            Access

Switch#show dtp

Switch#show interfaces trunk

Switch1(config)#interface fastEthernet 0/6

Switch1(config-if)#switchport trunk

Switch1(config-if)#switchport access

Switch1(config-if)#switchport mode dynamic desirable

Switch1(config-if)#switchport mode dynamic auto

VTP

SWITCH0(config)#vlan 2

SWITCH0(config-vlan)#name sales

SWITCH0(config-vlan)#exit

SWITCH0(config)#vlan 3

SWITCH0(config-vlan)#name marketing

SWITCH0(config-vlan)#exit

SWITCH0(config)#vlan 4

SWITCH0(config-vlan)#name Finance

SWITCH0(config-vlan)#exit

SWITCH0#show vlan

SWITCH0#show vtp status

SWITCH0(config)#vtp domain training

SWITCH0(config)#vtp mode server

SWITCH0(config)#vtp password cisco

SWITCH0(config)#end

SWITCH0#show vtp status

SWITCH1(config)#interface fastEthernet 0/1 #connected to switch0

SWITCH1(config-if)#switchport mode trunk

SWITCH1(config-if)#exit

SWITCH1(config)#interface fastEthernet 0/2 #connected to switch2

SWITCH1(config-if)#switchport mode trunk

SWITCH1#show vlan

SWITCH1#show vtp status

SWITCH1(config)#vtp domain training

SWITCH1(config)#vtp mode transparent

SWITCH1(config)#vtp password cisco

SWITCH1(config)#end

SWITCH1#show vtp status

SWITCH1#show vlan

Create a Vlan 5 (IT) and 6(HR)

SWITCH2#show vlan

SWITCH2#show vtp status

SWITCH2(config)#vtp domain training

SWITCH2(config)#vtp mode client

SWITCH2(config)#vtp password cisco

SWITCH2(config)#end

SWITCH2#show vtp status

SWITCH2#show vlan

try to create a new vlans, it will not allow

SERVER 				ONLY SERVER VLANS

TRANSPARENT				ONLY TRANSPARENT VLANS

CLIENT					ONLY SERVER VLANS

PORT SECURITY

switch#show mac address-table

switch#configure terminal

switch(config)#interface fastethernet 0/1

switch(config-if)#switchport mode access

switch(config-if)#switchport access vlan 1

switch(config-if)#switchport port-security

switch(config-if)#switchport port-security maximum 1

switch(config-if)#switchport port-security violation shutdown

switch(config-if)#switchport port-security mac-address 0005.5e05.5b0c

switch(config-if)#end

switch#show port-security

switch#show running-config

�� Violation Modes 3 options:

1.protect

Switch(config-if)# switchport port-security violation protect	No log message, no SNMP trap.

2.restrict

Switch(config-if)# switchport port-security violation restrict	Generates log message and SNMP trap.

3.shutdown (default)

Switch(config-if)# switchport port-security violation shutdown	Puts port In err-disabled state (administratively down).