Ipv4 addressing

32 bits address

4 0ctate

8.8.8.83

Class A				0XXXXXXX

Class B				10XXXXXX

Class C				110XXXXX

Class D				1110XXXX

Class E				1111XXXX

Ip address had two parts

Network id/address 			host id/address

Subnet mask

Class A				255.0.0.0

Class B				255.255.0.0

Class C				255.255.255.0

Class D				255.255.255.255

Class E

If you make all network bits one [ 1 ] then you get subnet mask.

And you write network bits from left to right and host bits from right to left

Logical and operation in ip address and subnet then you get network id

When all host address is 0 then it is the network address/id

If all host bits are one [ 1 ] then it is the broadcast ip for that network

Eg :- 1.255.255.255

Non routable ip address

Private ip vs public ip

Private ip ranges in each class

169.254.x.x 		 		automatic private IP address APIPA

Subnetting

Subnetwork

192.168.15.01 000000 				subnet address

192.168.15.01 000001				192.168.15.65

192.168.15.01 111110				192.168.15.126

192.168.15.01 111111				192.168.15.127

172.16.0.0/20

11111111.11111111.11110000.00000000

255.255.240.0

10.0.0.0/14

11111111.11111100.00000000.00000000

255.252.0.0

256-252 = 4

So range is 0,4,8,12….252

10.0.0.0/14

10.4.0.0/14

10.8.0.0/14

.

.

.

10.252.0.0/14

10.0.0.1/14   1st valid

10.3.255.254/14   last valid

10.3.255.255/14    broadcast

Flsm all this…

Fixed length subnet mask

Variable length subnet msk	[VLSM]

192.168.20.0/24

Day 2 cisco packet tracer 		Monday 		12-10-2026

Switch

Storage components

ROM – Firmware

RAM – running configuration [current configuration]

NVRAM – startup configuration [ changed configuration settings stored in the NVRAM ]

Flash – IOS

Booting Process

- ROM performs POST [ Power on self test ]

- Load the ISO from flash to RAM

- Load the configuration settings from NVRAM to RAM

- You will switch user mode.

Switch Modes

- User mode

- Privilege Mode

- Global configuration mode

- Specific configuration mode

  - VLAN

  - LINE

  - Interface

Other modes

- Setup mode		:	Initial configuration when start the switch/router

- ROMOM mode	:	While loading the IOS from flash to RAM [press CTRL + C to enter into rommon mode

  - We use this mode to recover password by setting register value to 0x2142 to skip NVRAM setup configuration

  - If IOS is corrupted you can restore from TFTP server

Tasks

- How to switch from one mode to another mode

- Use show command to check the RAM NVRAM and Flash memory content.

- Use copy command to copy the configuration from RAM to NVRAM, RAM to TFTP server, NVRAM to RAM, Flash to TFTP server, TFTP server to Flash.

- Set the password for console port , telnet access, privilege mode.

- Access the switch through console port, access the switch remotely using telnet and SSH

Press enter enter to view more commands

Day 3 oct 13

Show running-config

No ip domain-lookup 				command to ignore wrong command

telnet

Verify banner [through terminal or switch cli] and secret password [ when you change the mode from user priviledged mode]

Secret password override clear text password of priviledged level password

ROUTER

Tftp

Restoring ios from tftp to router

Refer pics from mobile

RECOVERING PASSWORD

Manually turn off and on the router and press ctrl+c while reloading to enter rommon mode

Further commands in mobile pics

DAY4 Network core 14 October

Routing

Default routing				iproute [network id] [subnet mask] [next hop]

Iproute 0.0.0.0 0.0.0.0 11.1.1.2

Static routing				R1	iproute 192.168.10.0 255.255.255.0 11.1.1.2

R2	Iproute 192.168.10.0 255.255.255.0 12.1.1.2							R2	Iproute 10.0.0.0 255.0.0.0 11.1.1.1

R3	Iproute 10.0.0.0 255.0.0.0 12.1.1.1

Dynamic routing			ínterior gateway routing protocols [single autonomous system]

RIP EIGRP[cisco proprietary] OSPF IS-IS

External gateway routing protocols

BGP

| RIP | EIGRP | OSPF |
| --- | --- | --- |
| 120 Administrative distance | 90 | 110 |
| Distance vector [bellman forte] | DUAL | djikstra |
| 15 hops supported | No limit | No limit |
| Hop count Cost metric | Bandwidth, delay, reliability,load mtl[max transition load] | bandwidth |
| Numbered hop formula | 256x(bandwidth+delay) | Reference bandwidth / Interface Bandwidth |

Classless scenario

Routing practicle

Commands in mobile pic

DYNAMIC ROUTING

RIP

Commands in mobile pic

EIGRP

OSPF ROUTING

Commands in mobile pics

DAY 5 October 15, 2026

ACL

Access control list

Allow or block packets based on IP, protocol and port no.

Removing configuration

Added server……..pic in mobile

Extended ACL

Pics in mobile

Task 3 TELNET

FTP

Block ftp commands in mobile

NAT [routing is required… so made copy of eigrp file]

DYNAMIC NAT

NAT overload PAT

DAY 6 OCTOBER 16

DNS

Port number 53

Tcp udp both

NSLOOKUP 		[ Name Server Lookup]

DNS Namespace

Ip should be assigned by dhcp

DNS server

Take pc that has ip from dhcp

Now in pc with static ip config

Route redistribution

September 19

VLAN

Save this file as vlan

AFTER BREAK

SOMETHING ELSE

Layer 3 switch vlan

DAY 8 		21 October

VTP

Refer to pics from mobile

Implementation not done properlu

3 vtp mode server transparent and client

Server 3 vlan

Transparant 2 vlans only 2 visible

Client only 3 from server visible

DTP

Refer mobile for all pics

Show interface trunk

PORT SECURITY

Refer mobile for all commands

STP 		SPANNING TREE PROTOCOL